Solar Car: Speeding on clean air

Abstract

Why are solar cars of interest in the EU and throughout the world?

In Cyprus, as in many other European countries, solar car races attracts people’s interest. Groups of students and other adults design and build their own cars to participate in these competitions. Furthermore, big car companies invest money and technology in future solar-powered cars.

This task aims to provide students with opportunities to discover the significance of adapting and using solar energy in transportation whilst exploring relevant mathematical and scientific content. Students complete a number of activities in mathematics, science and technology, applying knowledge from all three disciplines. Building on existing understanding and working on interdisciplinary activities, students will design, build and operate a model solar car.

http://commons.wikimedia.org/wiki/

File: 2009 Hunt-Winston Solar Car Challenge.jpg)

| Aspect | Description |
| --- | --- |
| Mathematical content | Proportionality, Functions, Statistics, Spatial Reasoning, Modelling |
| Scientific content | Gears, Power, Solar Energy |
| Age of students | 11– 14 |
| Number of lessons in mathematics | 5-6 |
| Number of lessons in science | 5-6 |

1. Overview of the teaching unit

## 1.1 The situation

The transport sector is the fastest growing consumer of energy and producer of greenhouse gases in the European Union despite advances in transport technology and fuel formulation that have resulted in marked decreases in emissions of certain pollutants.

Although both the environment and energy are issues in their own right, they clearly come together when looking at the subject of ‘transport sustainability’, since consumption and emissions are fairly closely linked: what goes into the fuel tank comes out of the exhaust pipe in the form of emissions.

EU countries are on the move. About 21% of the energy we use goes to transporting people and goods from one place to another. With the 83% share of transport energy consumed by road transport, this mode remains by far the largest single emitter, considering that more than 90% of the energy consumed comes from oil products. As a result, 93% of greenhouse gas emissions from transport came from road transport in 2004.

## 1.2 Guiding questions

- What are the main sources of air pollution?

- What is the impact of transport emissions on air pollution and the natural environment?

- What can we do to reduce air pollution coming from transport?

- To what extent can emissions be reduced if solar powered cars instead of petrol-powered cars are used (e.g. in a neighbourhood, in a town, in a country, in EU, etc.)?

## 1.3 Final product

At the end of the teaching unit students will design, build and operate a model solar car. While designing and building the car, students will study in depth the significance as well as the constraints of using (renewable) solar energy in transportation (and beyond), and explore mathematical and scientific knowledge in building and operating their model car.

The presentation of students’ results may take the form of PowerPoint presentations, webpage design, poster, written report, and/or a combination of the above.

## 1.4 Materials for experiment (activity 2)

- A materials kit, containing the following materials:

  - Small PV cells

  - Two electrical leads with alligator clips

- DC ammeter

- DC voltmeter

- Source of bright light or access to direct sunlight (desk lamp or flashlight could be substituted)

## 1.5 Suggestions

If car manufacturers in the participating countries put into production their own solar power cars (e.g., BMW, Mercedes, Seat), please adapt the materials to use EU vehicle models rather than Toyota’s.

1.6 The task plan

| Task Number | Mathematics | Science |
| --- | --- | --- |
| 1 | Task 1: Introduction to the context - How can renewable energy target the problem of air pollution? - How can solar power cars help reduce emissions? |  |
| 2 |  | Task 2: Power your Solar Car: Investigate the output of a Photovoltaic Unit (PV). How many photovoltaic units are needed to power a model solar car? Where is the best place to put the photovoltaic units? Students will carry out two investigations. At first, students investigate the relationship between the surface area of a photovoltaic unit and the power produced. Second, students explore the relationship between the angle of the light source to the power produced. |
| 3 | Task 3: Shape Up your Solar Car! Students will investigate how the tools provided in a dynamic geometry environment can be used to measure the surface area of their car. Students will explore how to measure the area of regular and irregular shapes and how to use / combine different views of a car to provide an estimate of its area. |  |
| 4 | Task 4: The Road from 2D to 3D! Solar Car Story! Investigate how connecting different 2D regular shapes can result in creating a net, which can then be folded to form the 3D shape of your Solar Car. |  |
| 5 |  | Task 5: Put your Car in Motion! Investigate gears of different type and size and put your car in motion. |
| 6 | Task 6: Your Solar Car Conquers the World! Run a survey analysing the sale price of your car and find the best selling price! Promote your car by highlighting its advantages. |  |
| 7 | Finalization: Solar Car is a Fact! Summarise information and developments in Solar Car design and production. |  |

2. Description of the tasks

2.1 Task 1 – Introduction to the overarching question

The aim of this task

- Introduction to the general context and framework.

Scientific content

- Energy sources (renewable and non-renewable) and air pollution

- Solar energy

- CO2 emissions

Outcomes

The main purpose of the introductory activity is to enable students to understand the role that solar cars might play in improving the quality of the environment. For this purpose, it is helpful for students to examine how cars using oil products contribute to air pollution.

Students are introduced to the problem of air pollution as a result of CO2 emissions from fuels used in transportation. Students further explore new commercial developments in the solar car industry and they finally get a more detailed insight into the use and operation of photovoltaic units.

Students will be presented with a set of articles retrieved and modified from the Web (see worksheet for task 1) presenting and commenting on the issues mentioned above. The introduction aims to provide a background for the rest of the activities and introduce students to relevant concepts.

|  | Activity 1 for students You are invited to read individually the articles presented in information sheet 1.1. Then discuss with your peers in groups of three (or four) the following questions: - What are the main causes of air pollution? What is the Greenhouse effect? - How large is the impact of transportation on total energy consumption? - What do CO2 emissions mean for the environment? How are these related to vehicle use? - How do photovoltaic units work? - How do Solar Cars work? - Make some notes in your group for a whole class discussion. One person will now stay in your group position, while the other members of your group will visit other groups and discuss your answers and ideas (around 15 minutes). Go back to your group and prepare a short poster (A4) on your core ideas (diagrams and other drawings are welcome). Alternatively you can prepare 2-3 PowerPoint slides. Each group will present their results to the whole class (25 minutes). |
| --- | --- |

2.2 Science – Task 2: Power your Solar Car: Investigate the Output of a Photovoltaic Unit

The aim of this task

The aim of the activity is twofold; first, to explore the relationship between the surface area of a photovoltaic unit and the electrical power generated, and second to explore the relationship between the angle of light and the photovoltaic (PV) unit and the power generated.

Contribution to the overall question

During the investigation students will familiarise themselves with core concepts for the Solar car task, like power, voltage, and current and will determine that PV surface area is linearly proportional to the power generated, while the relationship of the angle between light and PV unit is not linearly proportional, but is still proportional to the power generated.

Scientific content

- Power, voltage, current

- Assembly of simple circuits, using voltmeter and ammeter

Mathematical content

- Proportionality

- Plotting points, lines of best fit

- Functions

Outcomes

Students will explore properties of the PV unit, how its surface area is related to the power generated and what is the optimal angle of the light for the unit. Based on these studies, they can decide on the PV area needed for operating their solar car.

Materials

A materials kit, containing the following materials:

- small PV cells

- two electrical leads with alligator clips

- DC ammeter

- DC voltmeter

- Source of bright light or access to direct sunlight (desk lamp or flashlight could be substituted)

|  | Activity 1 for students Look at worksheets 2.1 “Photovoltaic Unit Surface” and 2.2 “Light and Photovoltaic Unit Angle” and try to complete the tasks, after constructing and running the experiments. Students design their own experiment in order to find the relationship between the surface area of a solar cell and the output power, using the kit provided. Students complete the table provided in Worksheet 1, plot the points on the corresponding graph and try to fit a curve to the points. It is expected that students will at least approach the relationship in a qualitative manner and determine that the relationship is linearly proportional. Further, some students might find a quantitative relationship and an algebraic formula. |
| --- | --- |

The provided graph is “empty” on purpose, in terms of axes, labelling and scaling. It is intended that students should develop their own scales and labels, before plotting points. If necessary, discuss their choices, but do not simply provide answers.

|  | Activity 2 for students In this activity students explore the relationship between the angle of incident light and the PV cell, and the output power, by running the experiment following the detailed instructions in Worksheet 2.2. |
| --- | --- |

Possible solutions

- As for Activity 1.

- It is expected that students should approach the relationship in a qualitative manner and determine that the relationship is not linearly proportional.

Teacher note:

- The relationship between the tangent of the angle of the incident light and the output power is proportional.

- It is intended that students should discuss the concept of light intensity.

|  | Optional activity for students: Open investigation In this activity students explore any other factors they believe might have an impact on the power of the PV unit. The investigation will be similar to those which took place in Activities 1 and 2. Students should complete worksheet 2.3. Possible solutions - It is expected that students should approach the investigation by controlling variables involved in the experiment and by examining one new variable and its relationship to the power of the PV unit (e.g., light intensity, light colour, etc.). |
| --- | --- |

2.3 Mathematics – Task 3: Shape up your Solar Car

The aim of this task

- How can we calculate the surface area of a car? How much material do we need for our model solar car?

- In this activity students calculate an estimate of the surface area of a real car. Students are provided with a number of 2D views of a car. By combining these 2D views using the tools provided in the environment of a Dynamic Geometry Software (DGS) to make measurements of user-defined areas, they will be able to estimate the surface area of the car.

Contribution to the overall question

Students find a method to calculate an estimate of the surface area of a 3D object. In this way, students find the surface area of their model solar car.

Mathematical content

- Measurements, 2D views of a 3D object

- Surface

- Regular and irregular polygons

Materials

In solving this activity students can copy the pictures provided in a Dynamic Geometry Software package and calculate the surface area of a car. For the purposes of this activity, Geogebra software can be used. Geogebra can be freely downloaded from www.geogebra.org, and it is available in all EU languages.

These pictures are found in the Car Shape.ggb file.

Activity 1 for students - Students are encouraged to find a method to calculate the total surface area of a car, based on a number of 2D images. (see worksheet 3.1)

- Students find a method to “combine” different images to construct a visual 3D image of the car.

- Students decide on how to scale the images provided.

- Students draw polygons (e.g., triangles, rectangles, other polygons) to measure the total surface area of a car.

- Based on their measurements and the scale factor, students calculate the surface area of the real car.

- Students estimate the surface area of their model solar car.

Pedagogy

Combining a mental 3D image based on various 2D images is a quite difficult task. Furthermore, it is important to underline to students that using polygons to measure the area of an irregular shape (the car) is exactly the same process used by professionals.

At first, it is important to clarify the concept of irregular shapes that appear in car 2D views. An irregular shape is one with sides of varying length. An irregular shape usually has four or more sides. Calculating the area of these shapes involves breaking the shape into regular shapes (e.g., triangles, squares, rectangles, circles).

Dynamic Geometry Tools

After exploring the process of “breaking” an irregular shape into regular shapes and calculating the surface area of the car, students can use the “polygon” tool in GeoGebra to draw irregular shapes that would cover the car. The software can automatically calculate the surface area of these shapes. Students can later compare the results from both investigations.

2.4 Mathematics – Task 4: The Road from 2D to 3D: The Solar Car Story

The aim of this task

Moving from 2D to 3D. In this activity students will work in a dynamic environment to create the 2D net for their model car. Then, they will fold the 2D net to construct a 3D model of their car. The purpose of this task is to enhance students’ spatial reasoning by connecting a 2D shape (comprised of a number of regular polygons) to a 3D one.

Contribution to the overall question

- Building the shape of the model solar car

Mathematical content

- Regular polygons (triangles, squares, rectangles)

- Nets

- Spatial reasoning

Materials

Software: Dalest Nets software can be freely downloaded from:

www.elica.net/site/museum/Dalest/dalest.html

Fig.1: Sample images representing 2D and a 3D view of a pyramid.

|  | Activity 1 for students - Time to build the shape of your model car! - Use Dalest Nets software to make a construction net from various regular polygons. Then fold your net to see your 3D model car. (Clipart from www.abcteach.com) |
| --- | --- |

This is a fairly challenging activity. It is important to advise students to use both the software and paper and pencil, to solve the problem.

An effective approach for students would be: (a) To draw a sketch of the car (3D model, like the one in the picture above) on paper. (b) Think of the 2D shapes needed to build the net that corresponds to the 3D model. (c) Use the software to design and fold the net. (d) Switch between the 2D and 3D view to make any necessary corrections and modifications.

Possible result:

A sample net and the corresponding 3D model may look like this.

Fig. 2: A sample net of the model car

Fig. 3: A sample 3D model of the car

2.5 Science – Task 5: Put your Car in Motion!

The aim of this task

In this activity students explore the different types of transmission and its implications for the design of their car. This activity provides opportunities for covering concepts from mathematics (e.g., proportionality), science (e.g., gears), and technology (e.g., types of gears).

Fig. 4: Different types of Gears

(Pictures from http://commons.wikimedia.org)

Contribution to the overall question

In this activity students consider the type of gears they will use in their model solar car. Students explore how gears are connected and the relationship between the number of teeth and gear speed.

Scientific content

- Gears, types of gears

Mathematical content

- Proportionality

Materials

Gears Applet

Fig. 5: The Gears applet

|  | Activity 1 for students - Students explore the relationship between the number of teeth in each gear and the rotating speed of the two gears. - Students work on the first screen of the applet provided. |
| --- | --- |

Pedagogy

The purpose of the applet is to help students identify the properties of gears (e.g., number of teeth, speed) and how they work in the context of a model car. If students have access to the materials kit, they will also build a model transmission for their car with mechanical parts. Alternatively, if the materials kit is not available, students can only perform the construction using the applet.

|  | Activity 2 for students - Students explore the relationship between the radius of each gear and its rotation speed, when the two gears are connected using a belt. - Students use the second screen of the applet provided. |
| --- | --- |

Pedagogy

The purpose of the applet is to help students identify the properties of gears (e.g., number of teeth, speed) and how they work in the context of a model car. If students have access to the materials kit, they will also build a model transmission for their car with mechanical parts. Alternatively, if the materials kit is not available, students can only perform the construction using the applet.

Fig. 6: The two types of Gears

2.6 Mathematics – Task 6: Your Solar Car conquers the world!

The aim of this task

In this activity students explore how the sale price of a car in relation to people’s interest has an impact on a company’s profits. Relating to management and economics, students are given opportunities to explore the mathematics of promoting and selling their car and connect their knowledge to a real world scenario.

Contribution to the overall question

In this activity students are given opportunities to explore a very significant aspect of vehicle manufacturing. The purpose of this activity is not directly related to the overall question (i.e., the solar power car), but it is strongly related to the idea of promoting the concept of using solar (and other renewable energy) cars.

Mathematical content

- Functions (linear and quadratic functions)

- Dynamic Geometry (car_price.ggb)

- Spreadsheets (data collection and presentation, including interactive graphs (linear, bar charts, etc.) and basic statistical functions.

Scientific content

Research skills, data coding and analysis

Materials

The introductory text presented in worksheet 6.1.

|  | Activity 1 for students Students explore the relationship between a car’s sale price and a company’s profits. The text informs students about a new solar car marketing policy and the results of a survey. ‘A new automobile company, located in EU, has recently launched a new solar powered car. The company ran a survey and the results are presented below: “A recent market research showed that one hundred people would buy the car for a selling price of 5000 euro. Further, the market research showed that for every 100-euro price increase, people interest in buying the car would decrease by one person.”’ Your task is to find the best selling price for the car, in order to maximise the company’s profits. |
| --- | --- |

Pedagogy

One of the purposes of the activity is to help students understand that it is not the highest price that returns the maximum profit. Furthermore, students will have opportunities to realise that the relationship between price and total money is not linear.

As suggested, an interesting extension of the activity is that students design their own questionnaire and distribute it to a number of people. For this purpose, students can use an online tool (www.surveymonkey.com) (free for the basic version) for designing and distributing their questionnaire. In this case, students can carry out the activity using the results of their questionnaire. In this case, students can focus on:

- learning the basic principles in collecting data using questionnaires

- designing questions for inclusion in the questionnaire (open questions, likert style, dichotomous questions, etc.)

- collecting and analysing data

- presenting data in tabular, figural, and verbal form

- constructing graphs

2.7 Finalisation – Solar Car is a fact!

The aim of this task

Combining all information and developments from previous tasks to present a comprehensive solution for a model solar car.

Outcomes

Students can present their results in different ways:

- A class discussion on the necessity of reducing CO2 and other greenhouse gas emissions and on the role and impact solar cars might have.

- Model car presentations.

- Car racing competition between model cars designed and constructed in this activity.

- Writing a newspaper article and/or presentation, advertising their solar car to potential buyers.